Cleaning Guidelines During Coronavirus (COVID 19)

for Historic Structures, Facilities and Materials

Effective Date: June 4, 2020

Most historic objects and surfaces cannot be safely cleaned or disinfected using common household cleaners such as furniture polishing sprays and countertop cleaners.  Many guidelines and websites recommend disinfecting surfaces with bleach or a bleach/water mixture.  Obviously, bleach will also heavily damage most historic objects.  The National Park Service (NPS) addresses these questions in COVID-19 Exhibitry Cleaning Guidance. It is recommended that all site staff and docents read these guidelines prior to June 12th.  All staff should follow these guidelines with particular attention to the section entitled “Exhibit Spaces are Open, but Virus Concerns Persist”.  Sites should follow the Georgia State Parks Cleaning Guidance for non-historic structures, facilities and materials found on Staff Resources.

- Post Visitor Center-Museum-Interpretive Center 2020 reopen sign found on Staff Resources on entrance doors, bathroom doors, bulletin boards etc.

- Consider temporarily restricting access to all hands-on activities and tactile exhibits. If some exhibit spaces have signs for tactiles that express “Please Touch”, remove these signs if possible, or temporarily cover them. Include on the signs that some historic objects are very difficult, if not impossible, to disinfect without causing irreversible damage to the object.

- Clean your tour routes as you normally would – sweep or vacuum floors, dust furniture and furnishings, clean exhibit cases with glass or plexiglass cleaner (depending on your cases materials). WypAll All-Purpose Wipes or Chix 8007 Soft Cloths are recommended cloths for cleaning glass and plexiglass. Plexiglass and glass display cases may need to be re-cleaned periodically throughout the day.

- Do not spray aerosol disinfectants, use “fog” treatments, or implement electrostatic guns in any exhibit spaces/areas. The effects of the chemicals contained in these treatments on materials is unknown.

- If there are areas where cleaning or disinfecting objects would be difficult to achieve easily signage, ropes, or other stanchions may be needed to limit access to areas by visitors.  Remember, visitors may still be able to see or look in areas while being prohibited from physically entering them.

- Clean wooden stair railings, tabletops, chairs or other surfaces that are repeatedly touched by visitors with an appropriate cleaner (see NPS guidelines for proper cleaning using the recommended Seventh Generation All-Purpose Cleaner – Free and Clear). Wipe back over the surface with a separate clean damp cloth to rinse excess cleaner off the surface.

- Always apply the cleaner to a rag or wipe first before cleaning a surface.  Never spray or apply the product directly onto the surface intended to be cleaned. Cloth micro-fiber rags are recommended for cleaning of most finished or smooth surfaces.  Avoid paper towels, which might abrade finishes or other delicate surfaces.

- When using a cleaning product, even those recommended in the guidelines, always start in a small, inconspicuous area on the object or exhibit case.  Wait a minute or two to check the area for any adverse reactions to the cleaning product before continuing to clean the object or exhibit case.  If any cracking, hazing, fading, or running of color takes place, do NOT continue to use the cleaner on that or any similar object.

- Remember, if an artifact is not touched, no risk of infection is warranted.  Special cleaners and disinfectants are not needed, and regular maintenance dusting or cleaning will suffice.

- If it appears that certain objects are prone to being repeatedly touched throughout the day, consider moving or isolating these objects to less accessible areas.

- Some objects are simply unable to be deep cleaned or disinfected without causing damage to them.  If an artifact is thought to be contaminated it should be isolated away from any human contact for 9 days at which point any virus contamination is considered neutralized.

- If the space/area has not been occupied for several days, the virus that was present on any surface is no longer viable. No additional cleaning or disinfecting is needed other than “normal” housekeeping tasks. Consult your Housekeeping Plan (see the NPS Museum Handbook Part 1, Chapter 13: Museum Housekeeping).

For any questions or concerns related to cleaning historic surfaces, artifacts, or cases at your museum or historic site, please contact:

Josh Headlee

GSPHS Preservation Specialist

Josh.Headlee@dnr.ga.gov