Recommended Cleaning Guidelines by Material Type               (5/29/2020)

Wood – There are a couple of different products that are highly recommended but should be used sparingly.  Repeated use of any product on wooden surfaces can erode and damage finishes and painted surfaces over time.  Even un-finished wood can be damaged by too much moisture.

Seventh Generation All Purpose Cleaner – Free and Clear is a good general cleaner (recommended by the National Park Service) that can be used on a variety of different surfaces including wood and metal.  The benefit to the Seventh Generation Free and Clear cleaner is that it is already mixed and can be used straight from the bottle.

Orvus WA Paste Cleaner is also a highly recommended all-purpose cleaning product that is safe to use on wood and textiles (upholstery).  The Orvus should be mixed with distilled water in the ratio of 1 tbsp. per gallon of water.  This solution should then be transferred to smaller squirt bottles (a small funnel comes in very handy).

Choose the above cleaner that best fits your needs (one also works well on upholstered furniture and textiles, the other can be used on metal and painted surfaces).  Apply the cleaner to a microfiber towel or WypAll All Purpose Wipe.  NEVER use a paper towel (including the brown, tri-fold variety) as they are too harsh and can scratch surfaces.  Wipe the dampened cloth over the wood using as few strokes as possible.  Never hold the cloth in one place for too long.  Using a second, clean, dampened with only water cloth, re-wipe the area cleaned to “rinse” away any excess cleaning product.  Using a third cloth or towel, dry the area.

Metal – Seventh Generation All Purpose Cleaner – Free and Clear can also be used sparingly on metal surfaces.    Rags or wipes moistened with rubbing or isopropyl alcohol can be used on most modern metal surfaces.  However, repeated use of alcohol can remove surface coatings and patina of some metals so, again, they should be used sparingly on historic metal objects.

*Note* - brass and copper (antique door knobs, etc.) naturally have anti-microbial properties.  This means that germs and viruses will naturally be killed within an 8-hour period.  This can help limit the amount of cleaning and disinfecting needed for these metal surfaces.

Textiles (Upholstery) – Avoid cleaning textiles unless absolutely necessary.  Most textiles should only be cleaned with a vacuum cleaner with regulatable suction power and the use of plastic screens. If more cleaning is required, a cleaning solution made up of 1 tbsp. of Orvus WA Paste cleaner can be mixed per gallon of distilled water.  Dampen and wring-out a cloth with the Orvus cleaning solution so that it is not dripping wet.  Dab the cloth onto the upholstery or textiles gently but quickly.  Afterwards, dry the textiles as much as possible by dabbing the surface again with a clean dry cloth.  When possible move any textiles or upholstered objects away from visitor contact to limit the amount of cleaning needed for these materials.

Leather – Leather can be cleaned in much the same way as textiles or upholstered furniture, but only if absolutely necessary and never on a frequently repeated schedule.  A soft sponge can be helpful in dabbing the Orvus solution onto leather.  If you notice any cracking or peeling, you should discontinue any further cleaning.  If, at all possible, leather objects should be moved away from visitor contact so that cleaning is not needed.

Plexiglass – Brillianize plastic and glass cleaner is recommended for cleaning all Plexiglass vitrines and surfaces.  Apply the Brillianize to a clean microfiber towel or Chix 8007 Soft Cloth to clean the plexiglass. NEVER use regular glass cleaner on plexiglass as it will cause irreversible damage to the surface such as hazing, fogging, or streaking.

Glass – Most glass display case fronts can be cleaned using any general glass cleaners.

Laminate (Formica, Wilson-Art) – Laminated display case enclosures or pedestals can be cleaned with most general all-purpose cleaners.  Brillianize also works well.  For harsher over-the-counter type cleaners, it is recommended that you apply a small amount with a cotton swab to an inconspicuous place on the laminate first to see if any color bleed or fading is likely to occur before cleaning the entire surface.

Supplies List

-WypAll All-purpose wipes - https://www.amazon.com/WypAll-34790CT-Cloths-POP-UP-White/dp/B0040ZOF9S/ref=sr_1_1?dchild=1&keywords=WypAll%2034790&qid=1588083978&sr=8-1

-Chix 8007 Soft Cloths - https://www.amazon.com/CLOTH-CHIX-SOFT/dp/B018H6WB8M

-Seventh Generation All Purpose Cleaner – Free and Clear - https://www.seventhgeneration.com/all-purpose-cleaner-free-clear

-Brillianize – plastic and glass cleaner - http://www.brillianize.com/

-Orvus WA Paste Cleaner - https://conservationsupportsystems.com/product/show/orvus-wa-paste/detergents-soaps - Orvus WA Paste Cleaner can also be found at most horse tack shops or tractor supply stores.

-Spray Bottles - https://www.uline.com/BL_6953/Spray-Bottles

-Isopropyl Alcohol - https://www.cvs.com/shop/cvs-health-91-isopropyl-alcohol-32-oz-prodid-1011920

-Alcohol Wipes - https://www.cvs.com/shop/cvs-health-isopropyl-rubbing-alcohol-70-wipes-40ct-prodid-1170225

-Distilled Water - https://www.globalindustrial.com/p/foodservice/food-beverages/coffee-beverages/distilled-water-1gal-6-carton?infoParam.campaignId=T9F&gclid=CjwKCAjw5cL2BRASEiwAENqAPh0QAtdflymz-yvZ_KNB8d2r4x6FPHm80AQgHoE4ztaQoNrJkhotMBoCBw4QAvD_BwE

-Vacuum Cleaner - https://www.vacuumcleanermarket.com/miele-classic-c1-pure-suction-sban0-vacuum-cleaner-parts.html

-Plastic Screening Material - https://www.lowes.com/pd/Phifer-BetterVue-3-ft-x-7-ft-Black-Fiberglass-Screen-Mesh/1000179077

-This cleaning guideline is based on the guidelines provided by the National Park Service - https://www.nps.gov/subjects/hfc/covid-19-exhibitry-cleaning-guidance.htm

If you have any questions or are not fully sure about cleaning or disinfecting any historic surfaces or artifacts at your site, please contact –

Josh Headlee

GSPHS Preservation Specialist

(404) 326-1534

Josh.Headlee@dnr.ga.gov

Quick Reference Cleaning Chart

| Wood | Seventh Generation All-Purpose Cleaner – Free And Clear | https://www.seventhgeneration.com/all-purpose-cleaner-free-clear | Pre-mixed – no mixing required | Apply product to wipe or soft clean cloth – use a rinse wipe – use a drying cloth |
| --- | --- | --- | --- | --- |
| Wood | Orus Wa Paste Cleaner | https://conservationsupportsystems.com/product/show/orvus-wa-paste/detergents-soaps | Mix 1 Tbsp. of Orvus Wa Paste Cleaner to 1 gal. of distilled water | Apply product to wipe or soft clean cloth – use a rinse wipe – use a drying cloth |
| Metal (Historic) | Seventh Generation All-Purpose Cleaner – Free And Clear | https://www.seventhgeneration.com/all-purpose-cleaner-free-clear | Pre-mixed – no mixing required | Apply product to wipe or soft clean cloth – use a rinse wipe – use a drying cloth |
| Metal (Brass and Copper) | NA | Has antimicrobial properties that will essentially disinfect itself after approximately 8-hours | NA | Can help reduce the amount of cleaning of historic brass |
| Metal (Modern Only) | Rubbing Alcohol or Alcohol Wipes | https://www.cvs.com/shop/cvs-health-91-isopropyl-alcohol-32-oz-prodid-1011920 or https://www.cvs.com/shop/cvs-health-isopropyl-rubbing-alcohol-70-wipes-40ct-prodid-1170225 | Pure alcohol can be diluted using distilled water de-pending on the percent-age of alcohol required | Wipe over the metal so as to fully sat-urate the surface |
| Textiles (Upholstery) | Vacuum (w/regulatable suction power) | https://www.vacuumcleanermarket.com/miele-classic-c1-pure-suction-sban0-vacuum-cleaner-parts.html | Use with a plastic screen as a barrier between the nozzle and fabric | Vacuum over the surface using as low of suction power as possible |
| Textiles (Upholstery) | Orvus Wa Paste Cleaner | https://conservationsupportsystems.com/product/show/orvus-wa-paste/detergents-soaps | Mix 1 Tbsp. of Orvus Wa Paste Cleaner to 1 gal. of distilled water | Blot or dab the Orvus solution onto the surface with a damp cloth – blot dry |
| Leather | Orvus Wa Paste Cleaner | https://conservationsupportsystems.com/product/show/orvus-wa-paste/detergents-soaps | Mix 1 Tbsp. of Orvus Wa Paste Cleaner to 1 gal. of distilled water | Same as with Textiles above but a soft sponge may be helpful |
| Plexiglass | Brillianize | http://www.brillianize.com/ | Pre-mixed – no mixing required | Apply product to wipe or soft cloth first. |
| Glass | Standard Glass Cleaners |  | Pre-mixed – no mixing required | Apply product to wipe or soft cloth first. |
| Laminate (Formica) | Brillianize or All-Purpose Cleaners | http://www.brillianize.com/ | Pre-mixed – no mixing required | Apply product to wipe or soft cloth first. |